The following checklist is to be reviewed for completion by the owner/contractor prior to scheduling an inspection.

Warning: Failure to complete all items on this checklist that pertain to your project may result in re-inspections, delays, and potential re-inspection fees.

Slab / Slab Gravel

- Slab on grade has 4” gravel, 6 mil polyethylene moisture barrier if required. IRC R506.2.3

- Vehicle bollard provisions for gas utilization equipment. IRC G2408.3 / G2408.4

- Reinforcing steel per plan.

- The tops of interior footing are cleared of sand and gravel. IRC R403

- String lines are pulled across forms to verify slab thickness. (Minimum 3 ½”) IRC R506.1

- Insulation requirements for heated floor slabs. IRC N1102.2.10 / (N11.01.1.2: edge insulation)

- Hydronic heat piping pressure test shall not be less than 100psi. IRC M2101.10